MOUSE Driver Software

The MOUSE driver software is an independently developed

driver program that enables you to operate programs which

were designed to be compatible with a Microsoft mouse.

MOUSE is compatible with programs such as Microsoft Word,

Borland's Reflex, Microsoft Codeview, PC Paintbrush,

Ventura Publisher, and many others.

An industry standard has evolved for mouse driven

programs, and every significant mouse based program uses a

separate interface program, called a mouse driver, to

communicate with the mouse. The mouse driver is a program

that is loaded once, after you load DOS, and it remains in

memory until you re-boot your computer or turn it off.

The mouse driver program's functions were originally

defined by MicroSoft. Z-Nix's MOUSE driver provides the

exact same functionality as Microsoft's mouse driver

MOUSE.COM. Different versions of MOUSE support different

hardware data formats.

The driver software has two main areas of functionality:

1) Returning "mouse" coordinates and button status to the

applications program.

2) Drawing and maintaining a tracking cursor that will

follow the motion of the tablet cursor or stylus.

Normally the operation of the driver will be transparent

to you. All you have to do is ensure that the driver is

loaded before you run the applications that use it.

Loading the MOUSE Driver:

MOUSE can be loaded two ways. The simplest way is to run

the program MOUSE.COM.

A>MOUSE

MOUSE will print a message confirming that it is loaded.

The driver defaults to selecting the first comm port that

it finds.

A command line option can be used to select which comm

port the mouse is connected to:

C>MOUSE /1   {for comm port 1}

C>MOUSE /2   {for comm port 2}

If you have a PS/2 system with extra serial ports you can

select comm3 or comm4 by using the option /3 or /4.

Four command options are provided to adjust the

sensitivity of the cursor tracking. The default

sensitivity setting provides the same tracking response

as a the mouse hardware. Each sensitivity option has a

number from 0 to 100 following it. 50 is the default

setting. Larger values make the tracking more rapid, lower

numbers reduce the sensitivity. Experiment to find a

comfortable setting.

The command options are:

/S  {sets both horizontal and vertical sensitivity}

/H  {sets the horizontal sensitivity}

/V  {sets the vertical sensitivity}

/D  {sets the speed threshold at which the tracking

rate doubles}

For example:

A>MOUSE /S100

makes a very speedy "mouse".

A>MOUSE /S10

provides a very slow "mouse", great for highly accurate

cursor pointing. These command line options can be

combined together, and they can also be issued after the

driver is initially loaded. For example, you can go back

and change the sensitivity of the cursor tracking with the

/S command option at any time.

Once the mouse driver is loaded into memory you are ready

to use your applications program. If you have a hard disk

drive you will probably want to copy MOUSE.COM onto your

hard disk. Since you only need to load MOUSE when you boot

up your computer many people add the MOUSE command to

their AUTOEXEC.BAT file so the driver is loaded

automatically whenever they start using their computer.

MOUSE will remain in memory, ready for use by any

applications program, until you either reboot or turn off

your computer. Remember though that if you reboot an

applications disk you will have to reload MOUSE. You can

disable the MOUSE driver by typing the command:

A>MOUSE /OFF

This command releases the comm port the mouse is connected

to.

The second way to load MOUSE is by including MOUSE.SYS as

a device driver in your CONFIG.SYS file. This method will

have a result that is identical to running MOUSE.COM each

time you boot up your computer.

Add the line below to your CONFIG.SYS file with a text

editor:

DEVICE=MOUSE.SYS

or if you need to use a command line option

DEVICE=MOUSE.SYS /V25

{for less vertical sensitivity}