EXT_UI DSP Flash Tool

V2.0

28-01-2002

Group Sense Technology (S) Pte Ltd

Content

1	Introduction	3

1.1	Function Performed by the TOOL	3

1.2	New Features added in V2.0	3

2	Specification	4

2.1	Equipment	4

2.2	Platform Supported	4

3	Constraint	5

3.1	Data Transfer	5

3.2	EXT_UI Reset	5

3.3	Other Constrain	5

4	Installation	6

5	Procedures/Layout	6

5.1	Preparation	6

5.2	Step 1: Select Files	7

5.3	Step 2: Flash EXTUI	8

5.4	Step 3: Write EXT_UI Product Information	10

6	Troubleshooting Guide	11

7	FBUS Message Window	12

# Introduction

This EXT_UI DSP flash tool (TOOL) sends data to EXT_UI board MCU (MCU) using FBUS via COM port, and MCU will in turn passing the data to the EXT_UI board DSP (DSP). This TOOL will be used as the DSP firmware upgrade in the field.

## Function Performed by the TOOL

- The TOOL will first read out EXT_UI board product information from the EXT_UI board.

- Then, the TOOL will transfer the INITDISK file to initialize the EXT_UI board’s NAND (NAND). After transferring the INITDISK file, the execution of the NAND initialization will take place, this takes about 20 seconds.

- Following that, the TOOL will transfer INITDATA file, which is the DSP executable code, to the NAND.

- After complete flashing, the EXT_UI board must be reset.  After reset, the TOOL will perform a SELFTEST (Same as the EXT_UI_TEST_SELF_TEST_REQ, refer to "Maverick FBUS message specification") for the EXT_UI board.

- Finally, the TOOL writes the EXT_UI board product information to the NAND.

The whole process for DSP firmware upgrading will take about 8 to 15 minutes.

## New Features added in V2.0

- No restriction for the location of INITDISK  and INITDATA files

- Able to continue flashing without re-start this TOOL

- User can change the delay used for NAND initialization (Refer to section 5.1)

- User can view FBUS message exchanges between PC and EXT_UI(Refer to section 5.1)

# Specification

## Equipment

Service accessory:			Type des:	Product code:

1. Flash Loading Adapter		BBS-8		0630261
2. DC Power Cable			PCS-1		0730012
3. Maverick UI Flash Cable		DAU-14	0730263

The service cable is connected to COM port of a PC. Generally a table top PC should be used instead of a laptop. Laptops sometimes have weak COM port drivers and since the service cable draws operating power from the port, unreliable operation may result.

8.0V operating voltage to the BBS-8 should be supplied either from a lab power supply or an FLA-7 flash loading adapter. If a lab power supply is used, it should give at least 2A current.

## Platform Supported

Win98, Win2000 and WinME

# Constraint

## Data Transfer

For normal FBUS message, if there is no RESP message from EXT_UI for the REQ sent by CMT, CMT will resent the REQ to EXT_UI.

However, during flashing, the MCU will disable the normal FBUS operation. It will not be possible for DSP to send flash status message to the TOOL during the process without modifying the MCU code. Therefore the TOOL will not be able to know whether the DSP has received the data and it will continue to transfer data to MCU.

This TOOL will assume that the MCU will always pass data to DSP successfully.  Only after section 1.1 (d), the unsuccessful flashing is detected.  This failure is generally due to DSP or NAND failure.

## EXT_UI Reset

During and after flashing, the MCU will stay in flash mode and normal FBUS function is disabled until the MCU being reset.  After reset, the TOOL will perform a SELFTEST to make sure that the EXT_UI is functioning, then writing the EXT_UI product information to the NAND.

To reset the EXT_UI, user may remove the phone from BBS-8 flash loading adapter for 5 seconds and then put it back.

## Other Constrain

Before running this TOOL, the PC should be configured in ALWAYS ON and screen saver in none mode .

During flashing, any PC tasks (like keyboard pressed, mouse click etc) may disrupt the data transfer between the TOOL and EXT_UI board. So, this TOOL disables the interrupt (like mouse click, key press) during the flashing process, in other word, PC will not perform any task, other than flashing.

# Installation

The installation package including EXTUI_Flash.CAB, Setup.exe, SETUP_LST, setup.txt and all support files.  To install, run setup.exe.  By default, it will be installed in C:\Program Files\flash1.  Make sure that flash.ini and flash.exe files are in the same folder.

# Procedures/Layout

## Preparation

- Check the default setting specified in FLASH.INI

Flash.ini stores four parameters as default setting with the parameter format: xxx="yyy". User can only modify the content inside the quotation mark.

(1) COMport Number. If the default COM port is not used, please change the port number in this file before running the flash.exe. Wrong COM port number will cause this tool not able to start.

(2) Format Time. This is the time used for NAND initialization. The default time is 20 seconds, which is sufficient for most phones. It can be varied from 20 to 59 seconds. Certain board may require longer time for NAND initialization, especially those not in good condition.  The default setting is the recommended setting and changes to this value to a shorter than the default may cause insufficient time to complete NAND initialization.

(3) Show Fbus Msg. If the user familiars with the FBUS Message protocol, set Show Fbus Msg="Yes" to view the FBUS Message (Refer to Section 7). This is useful for troubleshooting/fault analysis.

(4) The other default settings are the EXT_UI board product information. These will be used as default setting (Refer to section 5.4.a) if the TOOL is unable to read out the product information from EXT_UI board

Examples for changing the default settings:

Example1: change Serial Number="123456789" To Serial Number="987654321" Example2: change Show Fbus Msg="No" To Show Fbus Msg="Yes"

- Run the TOOL (Refer to Picture 1).

Picture1:Flash Tool Start

- Power on the phone set including the EXT_UI board

## Step 1: Select Files

- Select drive

- Select folder

- Select the INITDISK file used to initialize NAND

- Select the INITDATA file which is the program file

Example: refer to Picture 2, the files are in c:\Downloads, the INITDISK file is InitDisk_R058.a00 and the INITDATA file is InitData_R058_B3_pcb.a00

Picture 2: Step1:Select flash files

## Step 2: Flash EXTUI

- Press the “Step 2:Flash EXTUI” button to start flashing

After press this button, it will perform the following steps:

- Perform SELFTEST for the EXT_UI (Refer to Picture 3).

If the ERROR01 (Refer to Section 6) occurs, please reset the phone and flash again.

- Read out and display EXT_UI product information as shown in the Picture 4.

- Transfer INITDISK file to the EXT_UI board. Picture 4 shows that the INITDISK file transferring is completed.

- Wait for the execution of the NAND initialization. The time required is specified in flash.ini.

- Transfer INITDATA file to the EXT_UI board. Picture 5 shows that the INITDATA file transferring has completed. Phone is required to be reset in order to perform Step 3. To reset the phone, remove it from BBS-8 flash loading adapter for 5 seconds and then put it back. If the phone does not power up automatically in BBS-8, power button of the phone should be pressed.

| Picture 3: EXT_UI SELFTEST in Progress |  |
| --- | --- |
| Picture 4: Transfering INITDISK file completed |  |
| Picture 5: Flash Completed |  |

## Step 3: Write EXT_UI Product Information

- Please check and verify the product information before pressing “Step 3: Write EXT_UI product info” button.  If the production information file is not retrievable or NAND is new, the TOOL will not be able to read out the EXT_UI product information. User can manually fill in the product information or click "Default Setting" button to read from flash.ini.

- After the status window displaying “EXT_UI board is ready!”, press the "Step 3: Write EXT_UI product info" button to complete the whole flashing process. Picture 6 shows that it has successfully written the EXT_UI product information to EXT_UI board.

Before writing the product information to the EXT_UI, it will perform a SELFTEST for the EXT_UI (Refer to Picture 3). If the ERROR01 (Refer to Section 6) occurs, user may press this button again. If the ERROR01 has occurred for more than 3 times or one of ERROR02, ERROR03, ERROR04 and ERROR05 (Refer to Section 6) occurs, the EXT_UI flashing is unsuccessful. The user has to start from Step 2 again.

Picture 6: Write EXT_UI Product info successfully

# Troubleshooting Guide

| Error No. | Error description | Action to be taken |
| --- | --- | --- |
| Run-time error '8005' | port already open | - Please close the program using the default COM port. |
| ERROR01 | Fail to communicate with EXT_UI board! | - Make sure that phone is powered on and the DAU-14 service cable is connected to the default COM port. - Reset the phone and try again (Refer to Section 5.3.a and 5.4). MSG window (Refer to Section 7) will show how this error occurs. |
| ERROR02 | EXT_UI SelfTest Failed! | - Reset the phone and flash again (Refer to Section 5.4) |
| ERROR03 | EXT_UI DSP/NAND/AIC FAILED! |  |
| ERROR04 | EXT_UI Radio FAILED! |  |
| ERROR05 | EXT_UI USB FAILED! |  |
| ERROR06 | Wrong File format! | - User may have selected the wrong INITDISK or INITDATA file. |
| ERROR07 | FLASH EXT_UI_DSP failure! | - Please flash again. |
| ERROR08 | Hardware Version error in flash.ini! | - Please check it in the file flash.ini. Note: the number of characters inside the quotation mark must be 4. |
| ERROR09 | MCUSW Version error in flash.ini! | - Please check it in the file flash.ini. Note: the number of characters inside the quotation mark must be 11. |
| ERROR10 | DSPSW Version error in flash.ini! | - Please check it in the file flash.ini. Note: the number of characters inside the quotation mark must be 11. |
| ERROR11 | Serial Number error in flash.ini! | - Please check it in the file flash.ini. Note: the number of characters inside the quotation mark must be 9. |
| ERROR12 | Code error in flash.ini! | - Please check it in the file flash.ini. Note: the number of characters inside the quotation mark must be 7. |
| ERROR13 | Manufacture DDMMYY error in flash.ini! | - Please check it in the file flash.ini. Note: the number of characters inside the quotation mark must be 6. |
| ERROR14 | Invalid Manufacture DDMMYY in flash.ini! | - Please make sure that the date format is DDMMYY in flash.ini |
| ERROR15 | Invalid Manufacture DDMMYY! | - Please make sure that the date format is DDMMYY (Refer to Picture 6) |

# FBUS Message Window

If the user familiars with the FBUS Message (Refer to "Maverick FBUS message specification"), the FBUS message window (Picture 7) will allow user to view the FBUS message communication between PC and EXT UI board.  Note: FBUS message sent by CMT to EXT_UI will not be displayed.

To activate this MSG window, set Show Fbus Msg="Yes" in flash.ini before starting this TOOL.

Picture 7: FBUS Message Window

User guide for MSG window:

- User can type any message in the space at "MSG INPUT". The message should follow the FBUS message protocol: start with data3 (subfunction) and end before data L-2 (index number). All data are in Hex.

- Press button "SEND" to send the message at "MSG INPUT" to EXT_UI.

- Press button "PRINT MSG window" to print the whole MSG window.

- Press button "Clear" to clear the MSG window.

- The message following "<" is received from EXT_UI board and that following ">" is send to EXT_UI board by the TOOL.

-End-